goMoto 3.3 (Early 2020 – Present)

- Colours: Red and White

- Power: 1000W and 2000W models

- Top speed: 25 km/h and 45 km/h models

General information

60V 45Ah (5 x 12V 45Ah lead-acid)

This is new model, and the only bigger change is motor axle’s wormgear and radio. If you cannot fix the differential gear, you might need to order whole new rear axle. Difference between old and new model is the motor axle’s wormgear, old one has gear with diameter of 24mm and new one 28mm. That is also why you need matching rear axle (differential is integrated). Always measure the wormgear before ordering anything.

If you see radio with screws on top plate corners right under the windscreen, you are holding old type goMoto 3.3.

Technical specifications

| Length | 2260 mm |
| --- | --- |
| Width | 1160 mm |
| Height | 1510 mm |
| Wheelbase | 1700 mm |
| Seat height | 520 mm from ground level, 320 mm from floor level |
| Weight (including the batteries) | 240 kg |
| Top speed | 25 km/h / 45 km/h |
| Motors | 1000W or 1500W |
| Brakes | Hydraulic disc brakes |
| Battery capacity | 2,7kWh = 60V 45Ah (5 x 12V 45Ah) |
| Battery type | Lead-acid VRLA |
| Battery pack weight | 66 kg (5 x 13,2 kg) |
| Range | Up to 50 km |
| Charging time | Up to 12 hours, depending on the initial charge level of the batteries |
| Maximum load | 150,5 kg |
| Tyre size | Front = 130/60R13 ¦ Rear = 135/70R12 |
| Vehicle category | L2e-P |
| Number of seats | 2 |
| Seatbelt | Yes |
| Maximum torque of the motor | 48 N.m continuous |
| Maximum hill-starting ability at the maximum load | 8% slope |
| Available colors | Red and white |