Technical data

| F02 |  |
| --- | --- |
| Maximum design speed | 100km/h |
| Dimension |  |
| Full length | 1954mm |
| Full width | 703mm |
| Full height | 1280mm |
| Wheel Base | 1325mm |
| Seat height | 795mm |
| Min Ground Clearance | 141mm |
| Curb weight of whole vehicle | 143kg |
| Type | Centrally-mounted wheel rim and permanent magnet synchronous motor |
| Rated bus voltage | 72V |
| Rated power/peak power | 4kW/10kW |
| Rated speed | 4KW/715±15rpm |
| Rated torque | 53N.m/715±15rpm |
| Reduction ratio | 6.7 |
| Motor lubricating oil capacity | API GL-4 SAE 85W-9090mL |
| Charger |  |
| Input Voltage | 180 Vac~240 Vac |
| Rated Frequency | 50Hz〜60Hz |
| Charging current | 10A（900W） |
| Power Battery (single) |  |
| Battery Type | Ternary lithium battery |
| Nominal capacity | 40Ah x 2 pcs = 80Ah |
| Nominal Voltage | 72 V |
| Transmission |  |
| Transmission mode | Synchronous belt+aluminium pulley |
| Drive system | Motor Drive |
| Chassis |  |
| Tyre size | Front wheel 110/80-14 |
|  | Rear wheel 130/70-13 |
| Rim specification | Front wheel 2.50MTX14 |
|  | Rear wheel 3.50MTX13 |
| Electrical Installations |  |
| Headlight | High beam LEDX6 Low beam LEDX4 Front position lamp LEDX20 Front left and right headlight LEDX2 |
| Tail light | Brake light LEDX29 Rear left and right headlight LEDX2 Tail light LEDX29 |